Material Safety Data Sheet

Section 1 : Chemical Product and Company Identification

Product Name Ba-St Use of the substance PVC Stabilizer Company Identification WOOCHANG CHEMICAL Co,. Ltd. 223-34, Hwanseoung-li, Jigok-myeon, Seosan city, Chungnam, 356-861, Rep of Korea TEL : +82 41 664 6671 FAX : +82 41 664 6674

Section 2 : Composition, Information on Ingredients

Chemical Name Barium Stearate CAS Number 6865-35-6 Percentage Min. 99%

Section 3 : Hazards Identification

NFPA Ratings (Scale 0-4) Health=1, Fire=1, Reactivity=0 Emergency Overview White powder or crystalline solid. May form flammable or explosive dust-air mixtures. Avoid breathing dust. Avoid contact with eyes, skin and clothing. Avoid creation of dust. Wash thoroughly after handling. Use only with adequate ventilation. Keep container tightly closed. Potential Health Effects Inhalation Short Term Effects : May cause sore throat. Long Term Effects : No information available on significant adverse effects. Skin Contact Short Term Effects : May cause irritation. Long Term Effects : Same effects as short term exposure. Eye Contact Short Term Effects : May cause irritation. Long Term Effects : Same effects as short term exposure. Ingestion Short Term Effects: May cause ringing in the ears, stomach pain, dizziness and convulsions. Long Term Effects: No information available on significant adverse effects. Carcinogen Status OSHA : N, NTP : N, IARC : N

Section 4 : First Aid Measures

Inhalation Remove from exposure area to fresh air immediately. Perform artificial respiration if necessary. Maintain airway, blood pressure and respiration. Keep warm and at rest. Treat symptomatically and supportively. Get medical attention immediately. Qualified medical personnel should consider administering oxygen. Skin Contact Remove contaminated clothing and shoes immediately. Wash with soap or mild detergent and large amounts of water until no evidence of chemical remains (at least 15-20 minutes). Get medical attention immediately. Eye Contact Wash eyes immediately with large amounts of water or normal saline, occasionally lifting upper and lower lids, until no evidence of chemical remains (at least 15-20 minutes). Get medical attention immediately. Ingestion If vomiting occurs, keep head lower than hips to help prevent aspiration. Treat symptomatically and supportively. Get medical attention if needed. Note To Physician Antidote The following antidote has been recommended. However, the decision as to whether the severity of poisoning requires administration of any antidote and actual dose required should be made by qualified medical personnel. Poisoning From Barium Compounds Give 30 grams of sodium sulfate in 250 mL of water orally and repeat in one hour. Give by gastric tube if symptoms have appeared. The administration of sulfate salts intravenously is hazardous, since they induce the precipitation of barium sulfate in the kidney, with subsequent renal failure. Administration of potassium is critical.(Dreisbach, Handbook of Poisoning, 12th Ed.). Antidote should be administered by qualified medical personnel.

Section 5 : Fire Fighting Measures

Fire And Explosion Hazard Slight fire hazard when exposed to heat or flame. Dust-air mixtures may ignite or explode. Extinguishing Media Regular dry chemical, carbon dioxide, water, regular foam. For larger fires, use water spray, fog or regular foam. Firefighting Move container from fire area if you can do it without risk. Do not scatter spilled material with high-pressure water streams. Dike fire-control water for later disposal (1993 Emergency Response Guidebook, RSPA P5800.6, Guide Page 31). Use agents suitable for type of surrounding fire. Avoid breathing hazardous vapors, keep upwind. Flash Point: No data available. Lower Flammable Limit: No data available. Upper Flammable Limit: No data available. Autoignition: No data available. Hazardous Combustion Products: Thermal decomposition products may include toxic oxides of carbon.

Section 6 : Accidental Release Measuress

Occupational Spill Sweep up and place in suitable clean, dry containers for reclamation or later disposal. Do not flush spilled material into sewer. Keep unnecessary people away.

Section 7 : Handling and Storage

Observe all federal, state and local regulations when storing this substance. Store away from incompatible substances.

Section 8 : Exposure Controls / Personal Protection

Exposure Limits No occupational exposure limits established by OSHA, ACGIH, or NIOSH. Ventilation Provide local exhaust ventilation system. Ventilation equipment should be explosion-resistant if explosive concentrations of material are present. Eye Protection Employee must wear splash-proof or dust-resistant safety goggles to prevent eye contact with this substance. Emergency Eye Wash Where there is any possibility that an employee's eyes may be exposed to this substance, the employer should provide an eye wash fountain within the immediate work area for emergency use. Clothing Employee must wear appropriate protective (impervious) clothing and equipment to prevent repeated or prolonged skin contact with this substance. Gloves Employee must wear appropriate protective gloves to prevent contact with this substance. Respirator The following respirators are recommended based on information found in the physical data, toxicity and health effects sections. They are ranked in order from minimum to maximum respiratory protection. The specific respirator selected must be based on contamination levels found in the work place, must be based on the specific operation, must not exceed the working limits of the respirator and must be jointly approved by the National Institute for Occupational Safety and Health and the Mine Safety and Health Administration (NIOSHMSHA). Any dust and mist respirator. Any air-purifying respirator with a high-efficiency particulate filter. Any powered air-purifying respirator with a dust and mist filter. Any powered air-purifying respirator with a highefficiency particulate filter. Any type 'C' supplied-air respirator operated in the pressure-demand or other positive pressure or continuous-flow mode. Any self-contained breathing apparatus. For Firefighting And Other Immediately Dangerous To Life Or Health Conditions Any self-contained breathing apparatus that has a full facepiece and is operated in a pressure-demand or other positive-pressure mode. Any supplied-air respirator that has a full facepiece and is operated in a pressuredemand or other positive-pressure mode in combination with an auxiliary self-contained breathing apparatus operated in pressure-demand or other positive-pressure mode.

Section 9 : Physical and Chemical Properties

Description: White powder. Molecular Weight: 704 Molecular Formula: C36-H70-O4.Ba Boiling Point: Not applicable Melting Point: Not applicable Vapor Pressure: Not applicable Vapor Density: Not applicable Specific Gravity (Water=1): 1.1 Water Solubility: Insoluble pH: Not applicable Odor Threshold: Not available Evaporation Rate: Not applicable

Section 10 : Physical and Chemical Properties

Reactivity: Stable under normal temperatures and pressures. Conditions To Avoid May burn but does not ignite readily. Avoid contact with strong oxidizers, excessive heat, sparks, or open flame. Incompatibilities Oxidizing materials Oxidizers (Strong): Fire and explosion hazard. Hazardous Decomposition: Thermal decomposition products: oxides of sulfur, carbon. Polymerization: Hazardous polymerization has not been reported to occur under normal temperatures and pressures.

Section 11 : Toxicological Information

Toxicity Data 4 gm/kg oral-rat LD50 ; 3500 mg/kg oral-mouse LD50 ; 3600 mg/kg oral-guinea pig LD50 Carcinogen Status: None. Acute Toxicity Level: Moderately toxic by ingestion. Target Effects: Poisoning may affect the central nervous system. At Increased Risk From Exposure: Persons with a history of chronic respiratory, cardiovascular or skin disease.

Section 12 : Ecological Information

Environmental Impact Rating (0-4): No data available Acute Aquatic Toxicity: No data available Degradability: No data available Log Bioconcentration Factor (BCF): No data available Log Octanol/Water Partition Coefficient: No data available

Section 13 : Disposal Considerations

Observe all federal, state and local regulations when disposing of this substance.

Section 14 : Transport Informaiton

No classification currently assigned

Section 15 : Regulatory Information

U.S. Regulations : TSCA Inventory Status : Y CERCLA Section 103 (40CFR302.4) : N SARA Section 302 (40CFR355.30) : N SARA Section 304 (40CFR355.40) : N SARA Section 313 (40CFR372.65) : Y SARA Hazard Categories, SARA Sections 311/312 (40CFR370.21): Acute : Y Chronic : N Fire : N Reactive : N Sudden Release : N OSHA Process Safety (29CFR1910.119) : N EC Classification : No classification assigned

Section 16 : Addition Information

Date first edition : 2007. 12. 01 Date new edition : 2010. 05. 26 Product information See technical literature for details of suitable applications of this product. Additive for plastics. Note The purpose of the above information is to describe the products only in terms of health and safety requirements. The information given should no, therefore, be construed as guaranteeing specific properties or as a specification. Customers should satisfy themselves as to the suitability and completeness of such information for their own particular use.