Material Safety Data Sheet

Section 1 : Chemical Product and Company Identification

Substance Ca-ST Use of the substance PVC Stabilizer Company Identification WOOCHANG CHEMICAL Co., Ltd. 223-34, Hwanseoung-li, Jigok-myeon, Seosan city, Chungnam, 356-861, Rep of Korea TEL : +82 41 664 6671 FAX : +82 41 664 6674

Section 2: Hazards Identification

NFPA Ratings (Scale 0-4) Health=1 Fire=1 Reactivity=0 Emergency Overview: White powder. Distinct odor Major Health Hazards: No significant target effects reported. Physical Hazards: Dust/air mixtures may ignite or explode. Potential Health Effects: Inhalation Short Term Effects: Irritation. Long Term Effects: No information is available. Skin Contact Short Term Effects: No information is available. Long Term Effects: No information is available. Eye Contact Short Term Effects: Irritation. Long Term Effects: No information is available. Ingestion Short Term Effects: Digestive disorders. Long Term Effects: No information is available. Carcinogen Status: OSHA: N NTP: N IARC: N

Section 3 : Composition, Information on Ingredient

Ingredient Name Calcium stearate CAS No. 1592-23-0

Section 4 : First Aid Measures

Inhalation Remove from exposure immediately. Use a bag valve mask or similar device to perform artificial respiration (rescue breathing) if needed. Get medical attention. Skin Contact Remove contaminated clothing and shoes immediately. Wash with soap or mild detergent and large amounts of water until no evidence of chemical remains (at least 15-20 minutes). Get medical attention immediately. Eye Contact Wash eyes immediately with large amounts of water or normal saline, occasionally lifting upper and lower lids, until no evidence of chemical remains. Get medical attention immediately. Ingestion If vomiting occurs, keep head lower than hips to help prevent aspiration. Get medical attention if needed.

Section 5 : Fire Fighting Measures

Fire And Explosion Hazard Slight fire hazard when exposed to heat or flame. Dust-air mixtures may ignite or explode. Extinguishing Media Regular dry chemical, carbon dioxide, water, regular foam For larger fires, use regular foam or flood with fine water spray. Firefighting Move container from fire area if you can do it without risk. Avoid inhalation of material or combustion by-products. Stay upwind and keep out of low areas. Flash Point 351℉ (177℃) (OC) Lower Flammable Limit 0.025 g/L. Upper Flammable Limit No data available. Autoignition 752℉ (400℃).

Section 6 : Accidental Release Measuress

Occupational Spill Sweep up and place in suitable clean, dry containers for reclamation or later disposal. Do not flush spilled material into sewer. Keep unnecessary people away.

Section 7 : Handling and Storage

Observe all federal, state and local regulations when storing this substance. Store away from incompatible substances.

Section 8 : Exposure Controls / Personal Protection

Exposure Limits 5 mg/m3 OSHA TWA (respirable particulate) 15 mg/m3 OSHA TWA (total particulate) VENTILATION Provide local exhaust ventilation system. Ensure compliance with applicable exposure limits Eye Protection Wear splash resistant safety goggles. Provide an emergency eye wash fountain and quick drench shower in the immediate work area. Clothing Wear appropriate chemical resistant clothing. Gloves Wear appropriate chemical resistant gloves. Respirator Under conditions of frequent use or heavy exposure, respiratory protection may be needed. Respiratory protection is ranked in order from minimum to maximum. Consider warning properties before use. Any dust, mist, and fume respirator. Any air-purifying respirator with a high-efficiency particulate filter. Any powered, air-purifying respirator with a dust, mist, and fume filter. Any powered, air-purifying respirator with a high-efficiency particulate filter. For Unknown Concentrations or Immediately Dangerous to Life or Health Any supplied-air respirator with full facepiece and operated in a pressure-demand or other positive-pressure mode in combination with a separate escape supply. Any self-contained breathing apparatus with a full facepiece

Section 9 : Physical and Chemical Properties

Description White powder. Odor Distinct odor Boiling Point: Not applicable Melting Point 145~160℃ Vapor Pressure: Not applicable Vapor Density: Not applicable Specific Gravity (Water=1) 1.03 Water Solubility 0.004% @ 15℃ PH: Not applicable Odor Threshold: Not available Evaporation Rate: Not applicable Coefficient Of Water/Oil Distribution: Not available Solvent Solubility: Soluble: Hot pyridine Slightly Soluble: Hot alcohols, hot vegetable oils, mineral oils Insoluble: Chloroform, acetone, ether, cold alcohols

Section 10 : Physical and Chemical Properties

Reactivity Stable under normal temperatures and pressures. Conditions To Avoid Avoid heat, flames, sparks and other sources of ignition. Avoid contact with incompatible materials. Incompatibilities Oxidizing materials Oxidizers (Strong): Fire and explosion hazard. Hazardous Decomposition Thermal decomposition products: oxides of sulfur, carbon. Polymerization Will not polymerize.

Section 11 : Toxicological Information

Toxicity Data >10 gm/kg oral-rat LD50; >10 gm/kg oral-mouse LD50; >10 gm/kg intraperitoneal-mouse LD50; >1241 mg/m3/4 hour(s) inhalation-mammal LC Acute Toxicity Level: Insufficient Data.

Section 12 : Ecological Information

No information available

Section 13 : Disposal Considerations

Chemical waste generators must determine whether a discarded chemical is classified as a hazardous waste. Waste generators must consult local hazardous waste regulations to ensure complete and accurate classification.

Section 14 : Transport Informaiton

No information available

Section 15 : Regulatory Information

U.S. Regulations: TSCA Inventory Status: Y CERCLA Section 103 (40CFR302.4): N SARA Section 302 (40CFR355.30): N SARA Section 304 (40CFR355.40): N SARA Section 313 (40CFR372.65): Y SARA Hazard Categories, SARA Sections 311/312 (40CFR370.21): Acute: N Chronic: N Fire: N Reactive: N Sudden Release: N OSHA Process Safety (29CFR1910.119): N

Section 16 : Addition Information

The information above is believed to be accurate and represents the best information currently available to us. However, we make no warranty of merchantability or any other warranty, express or implied, with respect to such information, and we assume no liability resulting from its use. Users should make their own investigations to determine the suitable of the information for their particular purposes. We shall not be liable for any incidental or consequential damage arising directly or indirectly in connection with the purchase, use, storage or handling of this product. All recommendations or suggestions are made without guarantee, since the conditions of use are beyond our control. We disclaim any liability incurred in connection with the use of these data or suggestions. This information is not to be taken as a license to operate under, or a recommendation to infringe any patents. The observance of all regulations and patents is the responsibility of the user. No agent, representative or employee of this company is authorized to vary any terms of this notice.